(12230325)

Practical Activity #1: Using Wireshark to Examine Ethernet Frames

Part 1: Examine the Header Fields in an Ethernet II Frame

Q1: What is significant about the contents of the destination address field?
The destination address field can be either a unicast or a broadcast address. In the case of a broadcast (ff:ff:ff:ff:ff:ff), the frame is intended for all devices on the local network segment1.

Q2: Why does the PC send out a broadcast ARP prior to sending the first ping request?
The PC sends a broadcast ARP to discover the MAC address of the default gateway, as it needs this information to send packets outside its local subnet1.

Q3: What is the MAC address of the source in the first frame?
The MAC address of the source is f4:8c:50:62:62:6d1.

Q4: What is the Vendor ID (OUI) of the Source’s NIC?
The Vendor ID (OUI) is the first three octets: f4:8c:501.

Q5: What portion of the MAC address is the OUI?
The OUI is the first 6 hexadecimal digits (first 3 octets) of the MAC address1.

Q6: What is the Source’s NIC serial number?
The serial number is the last 6 hexadecimal digits: 62:62:6d1.

Part 2: Use Wireshark to Capture and Analyze Ethernet Frames

Q7: What is the IP address of the default gateway for the host H3?
The default gateway for H3 is 10.0.0.11.

Q8: What is the MAC address of the PC’s NIC?
This can be found using ip address on H3; assume it is 5a:d0:1d:01:9f:be (as shown in the ARP cache example)1.

Q9: What is the default gateway’s MAC address?
The gateway’s MAC address is the one associated with 10.0.0.1 in the ARP cache, which you would see in Wireshark or ARP output1.

Q10: What type of frame is displayed?
The frame type is Ethernet II1.

Q11: What is the source IP address?
The source IP address is the IP of H3 (e.g., 10.0.0.11)1.

Q12: What is the destination IP address?
The destination IP address is the gateway (10.0.0.1) or the remote host, depending on the ping target1.

Q13: What device and MAC address is displayed as the destination address in the Echo reply?
The destination device is H3, and the MAC address is 5a:d0:1d:01:9f:be1.

Q14: In the first echo (ping) request frame to 172.16.0.40, what are the source and destination MAC addresses?

- Source: H3’s MAC address (e.g., 5a:d0:1d:01:9f:be)

- Destination: Gateway’s MAC address (since the destination is not on the local subnet)1.

Q15: What are the source and destination IP addresses contained in the data field of the frame?

- Source: H3’s IP address (e.g., 10.0.0.11)

- Destination: 172.16.0.401.

Q16: Why has the destination IP address changed, while the destination MAC address remained the same?
The destination IP changes because the ping is now targeting a remote host, but the destination MAC remains the gateway’s MAC because the packet must be sent to the gateway for routing outside the local network1.

Q17: What does the preamble contain?
The preamble contains synchronizing bits, processed by the NIC hardware, and is not shown in Wireshark captures1.